# V2L Relay Controller — Design Review & PCB Layout Guidelines

Date: July 14, 2026
Project: V2L-Relay-Controller
Reviewer: AI-assisted design review (Kiro)
Target: Intern implementing PCB trace layout

## 1. Executive Summary

Is this doable on a 2-layer PCB? Yes — but with important caveats.

This board routes only 12V DC relay coil current and 3.3V logic signals on the PCB copper itself. The 120/240V AC passes through the relay contacts and exits via screw terminals — the high-voltage path is external wiring connected to the terminal blocks, not PCB traces. This makes the design fundamentally a low-voltage (12V/3.3V) relay driver board, which is well-suited to 2 layers.

The challenges are: - Large component count (~120 parts) over a wide footprint - 16 relay coils drawing ~40mA each (total ~640mA on +12V bus) - Adequate GND return path on a 2-layer board with many through-hole parts - Creepage/clearance around the screw terminals where external 120/240V wires connect

## 2. Critical Design Rule Corrections (Do This First)

The current design rules have 0mm clearance — this is invalid and will cause manufacturing failures.

| Parameter | Current Value | Required Minimum | Recommended |
| --- | --- | --- | --- |
| Copper clearance | 0 mm | 0.15 mm | 0.25 mm |
| Min track width | 0.2 mm | 0.15 mm | 0.25 mm (signals), 0.5-1.0 mm (power) |
| Via diameter | 0.6 mm | 0.5 mm | 0.6 mm ✓ |
| Via drill | 0.3 mm | 0.25 mm | 0.3 mm ✓ |

For screw terminal pads (where external AC wires connect): Enforce 2.5 mm clearance between the terminal pads and any adjacent low-voltage copper. This provides basic creepage for the external 120/240V connection points, per IPC-2221B for ≤250V working voltage.

Action in KiCAD: 1. Set global clearance to 0.25 mm 2. Create a net class “HV_Terminals” with 2.5 mm clearance and assign the screw terminal nets 3. Create a net class “Power_12V” with 0.5 mm track width for +12V and GND bus traces

## 3. Track Width Guidelines

| Net / Purpose | Current (max) | Recommended Width | Notes |
| --- | --- | --- | --- |
| AC power (screw terminal to relay contact) | 10 A | 3.0-5.0 mm | Highest current path! No vias, no layer changes |
| +12V bus (main spine) | ~640 mA total | 1.0 mm | Sum of all 16 relay coils + LEDs |
| +12V branches (per relay) | ~40 mA | 0.5 mm | Individual relay coil feed |
| GND bus (main spine) | ~640 mA return | 1.0 mm | Use copper pour where possible |
| /SW_12V (switched 12V) | ~40 mA per switch | 0.5 mm | Manual override supply |
| 3.3V (LDO output) | <50 mA total | 0.4 mm | Optocoupler pull-ups + STM32 |
| Signal traces (PA/PB to MOSFETs) | <1 mA | 0.25 mm | Logic signals, noise-tolerant |
| LED current-limiting paths | ~15 mA | 0.3 mm | Between resistor and LED |

Temperature Rise Rule of Thumb: On outer copper layer (1 oz/ft²), 1mm width handles ~1A with <10°C rise. Your maximum bus current of ~640mA is well within 1mm trace capability.

## 4. Board Outline & Size Recommendation

Current component placement spans roughly 455mm x 165mm — that’s very large. With careful rearrangement:

Recommended board size: 250mm x 120mm (approximately 10” x 5”)

This is achievable because: - The 16 relay channels are repetitive subcircuits that can be packed in a regular grid - The G5Q-1A relay footprint is ~10.7mm x 21.6mm — 16 relays in two rows of 8 takes ~180mm x 55mm - Driver circuits (BJT + MOSFET + opto + diode + switch + LED) fit in ~15mm x 25mm per channel

If cost/size is not a concern, the current spread-out placement is fine for prototyping — just add an Edge.Cuts rectangle around the placed components with 3-5mm margin.

## 5. Component Placement Strategy

### Zone Layout (recommended arrangement)

┌─────────────────────────────────────────────────────────────┐
│  SCREW TERMINALS (top edge)                                  │
│  ─────────────────────────────────────────────────────────── │
│  RELAY ROW 1 (K19-K30, 8 relays + flyback diodes)           │
│  ─────────────────────────────────────────────────────────── │
│  DRIVER ROW 1 (BC337 + 2N7000 + opto + switch + LED x 8)    │
│  ═══════════════════════════════════════════════════════════  │
│  RELAY ROW 2 (K31-K34 + K20-K22, 8 relays + flyback diodes) │
│  ─────────────────────────────────────────────────────────── │
│  DRIVER ROW 2 (BC337 + 2N7000 + opto + switch + LED x 8)    │
│  ─────────────────────────────────────────────────────────── │
│  SCREW TERMINALS (bottom edge)                               │
│                                                              │
│  [STM32 BluePill]  [LDO + caps]  [Input connector J12/COM]  │
│  (center/right)                                              │
└─────────────────────────────────────────────────────────────┘

### Placement Rules

- Keep each relay’s driver subcircuit together — the BC337, 2N7000, optocoupler, flyback diode, switch, and LED for one relay should form a tight cluster adjacent to that relay.

- Screw terminals at board edges — these carry external AC wires and need easy access for the user. Place them along the top or bottom edge with at least 5mm clearance to any other copper.

- STM32 BluePill module in a central/protected location — away from the relay coils (which generate EMI when switching). At least 15-20mm separation from the nearest relay.

- LDO and decoupling caps (IC1, C1, C2, C7-C10) — place physically close to the STM32 module, with the 0.1uF caps within 5mm of the 3.3V pin.

- Align repetitive subcircuits — identical relay driver circuits should have identical relative placement. This makes routing much faster (route one channel, then replicate the pattern).

- Switches and LEDs — place near the board edge for user access. The slide switches (manual override) and status LEDs should face the operator.

## 6. Grounding Strategy (Critical for 2-Layer)

On a 2-layer board, you don’t have a dedicated ground plane. This requires deliberate grounding:

- Use a B.Cu (back copper) ground pour — Fill the entire back layer with a GND copper zone. This acts as a pseudo-ground-plane and provides excellent return paths.

- Star-ground topology for front copper:

  - Run a thick (1.5mm) GND bus spine along the length of the board

  - Branch to each relay driver subcircuit with individual 0.5mm GND traces

  - Keep the STM32/LDO ground connected to the main bus via a short, wide trace

- Stitching vias — Place GND vias every 15-20mm connecting F.Cu ground traces to the B.Cu pour. This keeps the ground return path short.

- Do NOT route signal traces on B.Cu if possible — Reserve the back layer primarily for the ground pour. Only use B.Cu traces for short jumpers where absolutely necessary to avoid congestion.

- Separate analog/digital ground connection — The optocoupler emitters connect to local ground. Ensure these return currents don’t flow through the STM32’s ground path. The star topology handles this naturally.

## 7. Routing Priority Order

Route in this order (highest priority first):

- GND connections — tie all ground pads, then pour B.Cu ground zone

- +12V bus — thick trace spine with branches to each relay coil and driver

- /SW_12V — switched 12V to the manual override switches

- 3.3V power (LDO to STM32, pull-up resistors) — short traces near the controller

- Relay coil connections — relay pin to flyback diode to BC337 collector

- Driver signal chain — STM32 pin to 2N7000 gate to BC337 base / optocoupler paths

- LED indicator paths — lowest priority, can tolerate longer routes

- Screw terminal connections — relay contact pins to terminals (keep short and direct)

## 8. Specific Design Concerns & Recommendations

### A. AC Power Traces (Screw Terminal to Relay Contact)

- These traces carry up to 10A at 120/240V AC — they are the highest-current, highest-voltage paths on the board

- Minimum trace width: 3.0 mm (prefer 4-5mm where space allows)

- Route entirely on F.Cu — no vias, no layer transitions in the AC current path

- Use 45-degree angles only — no sharp 90° corners (current crowding + potential arc point)

- Maintain 2.5 mm clearance to all adjacent copper (IPC-2221B creepage for 240V)

- Keep these traces as short and direct as possible — place each screw terminal immediately adjacent to its relay

- Consider using copper fills/zones instead of traces for these connections — a filled polygon between the terminal pad and relay pad handles current and heat better than a narrow trace

- Do NOT route any low-voltage signal traces between or parallel to these AC traces

### B. Flyback Diodes (D4-D28, 1N4007)

- Place each diode directly across its relay coil pins — within 5mm

- Route cathode to +12V and anode to relay coil with short, wide (0.5mm) traces

- The 1N4007 is through-hole (DO-41 package) — orient all diodes consistently for easy inspection

### C. Optocoupler Input/Output Isolation

- The LTV-817C provides galvanic isolation between external control signals and the relay drivers

- Keep the input side (pins 1-2, anode/cathode) traces separate from output side (pins 3-4, emitter/collector)

- Do NOT run input and output traces in parallel — maintain >=1mm separation

### D. 2N7000 MOSFET Gate Drive

- These are logic-level MOSFETs driven by the STM32 (3.3V)

- The gate traces carry essentially zero current — 0.25mm is fine

- The 10k pull-down resistors (R8-R11) should be placed close to each MOSFET gate

- Keep gate traces short to minimize noise pickup

### E. Decoupling Capacitors

- C7, C8, C9, C10 (0.1uF) and C1 (2.2uF), C2 (1uF) are for the LDO

- Place 0.1uF caps within 3-5mm of the power pins they protect

- Route directly from cap pad to power pin, not through a via if possible

### F. Screw Terminal Clearance

- The 1729128 screw terminals handle external 120/240V AC wiring

- On the PCB, enforce 2.5mm minimum clearance between terminal pads and adjacent copper

- Consider adding milled slots (cutouts in the PCB) between the terminal area and the rest of the board for enhanced creepage — though this is optional for a prototype

- Add silkscreen warnings: “DANGER: HIGH VOLTAGE” near terminals

## 9. Manufacturing Notes

| Parameter | Recommendation |
| --- | --- |
| Copper weight | 1 oz/ft² (35um) — sufficient for 640mA max |
| Board thickness | 1.6mm standard |
| Minimum hole size | 0.3mm (for vias) — already set correctly |
| Solder mask | Both sides — essential for preventing shorts |
| Silkscreen | Front side minimum; both sides recommended |
| Surface finish | HASL (leaded or lead-free) — cheapest, fine for THT |
| Panelization | Not needed for single prototype |

## 10. Checklist Before Routing

- Set global clearance to 0.25mm

- Create net class “Power” (track 1.0mm) and assign +12V, GND

- Create net class “HV_Terminal” (clearance 2.5mm) and assign screw terminal nets

- Draw board outline on Edge.Cuts layer

- Add 4 mounting holes (M3, corner positions)

- Verify all component footprints match physical parts

- Run DRC after placing outline (fix 0mm clearance violations)

- Place B.Cu ground pour covering full board area

- Route power bus first, then signals

- Add GND stitching vias (every 15-20mm grid)

- Run final DRC — zero errors before ordering

## 11. Verdict

2-layer PCB: FEASIBLE and appropriate for this design.

Rationale: - No high-speed signals (relay switching is <1kHz, STM32 signals are GPIO only) - No sensitive analog circuits - Maximum current per trace is modest (~640mA bus, ~40mA branches) - The AC high-voltage path is external — it passes through the relay contacts and screw terminals, not through PCB traces - The repetitive nature of 16 identical channels makes routing systematic

The only scenario where 4 layers would be justified is if you needed much tighter board dimensions or EMC certification. For a prototype control board, 2 layers is the right choice.

## Component Reference Quick-Lookup

| Component Group | References | Quantity |
| --- | --- | --- |
| Relays (G5Q-1A-DC12) | K19-K34 | 16 |
| Flyback diodes (1N4007) | D4-D28 | 16 |
| Relay drivers (BC337) | Q8, Q13-Q14, Q23, Q25, Q27-Q28, Q52-Q60 | 16 |
| Level shifters (2N7000) | Q6-Q7, Q9-Q10, Q15-Q18, Q26, Q29-Q31, Q40-Q43 | 16 |
| Optocouplers (LTV-817C) | Q3, Q5, Q11-Q12, Q19-Q22, Q32-Q35, Q44-Q47 | 16 |
| Screw terminals (1729128) | J5-J12, J19-J29 | 16+ |
| Slide switches (JS102011CQN) | S1, S3-S18 | 18 |
| LEDs (SML-D12M8WT86) | LED1-LED18 | 18 |
| STM32 BluePill | U1 | 1 |
| 3.3V LDO (TLV76133) | IC1 | 1 |
| Mode switch (100SP1T1B4M2QE) | S2 | 1 |